# BLOCK DIAGRAM

The block diagram illustrates the working principle of the ESP32-based power monitoring and control system:

- Power Measurement using PZEM-004T

  - The PZEM-004T module measures real-time AC voltage, current, power, and energy consumption of the connected load.

  - This data is transmitted to the ESP32 via UART communication.

- ESP32 Microcontroller

  - Acts as the main control unit.

  - Receives power consumption data from PZEM-004T.

  - Controls two relays to switch loads ON/OFF.

  - Connects to Wi-Fi and communicates with Adafruit IO Cloud for remote monitoring.

- Relay Control for Load Management

  - Two relays are used to turn ON/OFF connected AC loads based on user input or automation logic.

  - This enables load segregation and power factor optimization, reducing penalties for excessive reactive power.

- Wi-Fi and Cloud Integration

  - The ESP32 connects to the internet via Wi-Fi.

  - It sends real-time power data to Adafruit IO Cloud, allowing users to monitor power usage remotely.

  - Users can send control signals from a web dashboard or mobile app to toggle relays.

- Power Supply Unit

  - Ensures proper voltage levels to power the ESP32, PZEM-004T, and relay modules.

This system provides real-time power monitoring, remote control of loads, and cost-effective power factor management, making it suitable for industrial and commercial applications.