## Introduction to ESP32

The ESP32 is a series of chip microcontrollers developed by Espressif.

Why are they so popular? Mainly because of the following features:

- Low-cost: you can get an ESP32 starting at $6, which makes it easily accessible to the general public;

- Low-power: the ESP32 consumes very little power compared with other microcontrollers, and it supports low-power mode states like deep sleep to save power;

- Wi-Fi capabilities: the ESP32 can easily connect to a Wi-Fi network to connect to the internet (station mode), or create its own Wi-Fi wireless network (access point mode) so other devices can connect to it—this is essential for IoT and Home Automation projects—you can have multiple devices communicating with each other using their Wi-Fi capabilities;

- Bluetooth: the ESP32 supports Bluetooth classic and Bluetooth Low Energy (BLE)—which is useful for a wide variety of IoT applications;

- Dual-core: most ESP32 are dual-core— they come with 2 Xtensa 32-bit LX6 microprocessors: core 0 and core 1.

- Rich peripheral input/output interface—the ESP32 supports a wide variety of input (read data from the outside world) and output (to send commands/signals to the outside world) peripherals like capacitive touch, ADCs, DACs, UART, SPI, I2C, PWM, and much more.

- Compatible with the Arduino “programming language”: those that are already familiar with programming the Arduino board, you’ll be happy to know that they can program the ESP32 in the Arduino style.

- Compatible with MicroPython: you can program the ESP32 with MicroPython firmware, which is a re-implementation of Python 3 targeted for microcontrollers and embedded systems.

### ESP32 Specifications

If you want to get a bit more technical and specific, you can take a look at the following detailed specifications of the ESP32 (source: http://esp32.net/)—for more details, check the datasheet):

ESP32 module: ESP-WROOM-32

- Wireless connectivity WiFi: 150.0 Mbps data rate with HT40

  - Bluetooth: BLE (Bluetooth Low Energy) and Bluetooth Classic

  - Processor: Tensilica Xtensa Dual-Core 32-bit LX6 microprocessor, running at 160 or 240 MHz

- Memory:

  - ROM: 448 KB (for booting and core functions)

  - SRAM: 520 KB (for data and instructions)

  - RTC fast SRAM: 8 KB (for data storage and main CPU during RTC Boot from the deep-sleep mode)

  - RTC slow SRAM: 8KB (for co-processor accessing during deep-sleep mode)

  - eFuse: 1 Kbit (of which 256 bits are used for the system (MAC address and chip configuration) and the remaining 768 bits are reserved for customer applications, including Flash-Encryption and Chip-ID)

  - Embedded flash: flash connected internally via IO16, IO17, SD_CMD, SD_CLK, SD_DATA_0 and SD_DATA_1 on ESP32-D2WD and ESP32-PICO-D4.

    - 0 MiB (ESP32-D0WDQ6, ESP32-D0WD, and ESP32-S0WD chips)

    - 2 MiB (ESP32-D2WD chip)

    - 4 MiB (ESP32-PICO-D4 SiP module)

- Low Power: ensures that you can still use ADC conversions, for example, during deep sleep.

- Peripheral Input/Output:

  - peripheral interface with DMA that includes capacitive touch

  - ADCs (Analog-to-Digital Converter)

  - DACs (Digital-to-Analog Converter)

  - I²C (Inter-Integrated Circuit)

  - UART (Universal Asynchronous Receiver/Transmitter)

  - SPI (Serial Peripheral Interface)

  - I²S (Integrated Interchip Sound)

  - RMII (Reduced Media-Independent Interface)

  - PWM (Pulse-Width Modulation)

- Security: hardware accelerators for AES and SSL/TLS

### Main Differences Between ESP32 and ESP8266

Previously, we mentioned that the ESP32 is the ESP8266 successor. What are the main differences between ESP32 and ESP8266 boards?

The ESP32 adds an extra CPU core, faster Wi-Fi, more GPIOs, and supports Bluetooth 4.2 and Bluetooth low energy. Additionally, the ESP32 comes with touch-sensitive pins that can be used to wake up the ESP32 from deep sleep, and built-in hall effect sensor.

Both boards are cheap, but the ESP32 costs slightly more. While the ESP32 can cost around $6 to $12, the ESP8266 can cost $4 to $6 (but it really depends on where you get them and what model you’re buying).

So, in summary:

- The ESP32 is faster than the ESP8266;

- The ESP32 comes with more GPIOs with multiple functions;

- The ESP32 supports analog measurements on 18 channels (analog-enabled pins) versus just one 10-bit ADC pin on the ESP8266;

- The ESP32 supports Bluetooth while the ESP8266 doesn’t;

- The ESP32 is dual-core (most models), and the ESP8266 is single core;

- The ESP32 is a bit more expensive than the ESP8266.

For a more detailed analysis of the differences between those boards, we recommend reading the following article: ESP32 vs ESP8266 – Pros and Cons.

## ESP32 Development Boards

ESP32 refers to the bare ESP32 chip. However, the “ESP32” term is also used to refer to ESP32 development boards. Using ESP32 bare chips is not easy or practical, especially when learning, testing, and prototyping. Most of the time, you’ll want to use an ESP32 development board.

These development boards come with all the needed circuitry to power and program the chip, connect it to your computer, pins to connect peripherals, built-in power and control LEDs, an antenna for wi-fi signal, and other useful features. Others even come with extra hardware like specific sensors or modules, displays, or a camera in the case of the ESP32-CAM.

### How to Choose an ESP32 Development Board?

Once you start searching for ESP32 boards online, you’ll find there is a wide variety of boards from different vendors. While they all work in a similar way, some boards may be more suitable for some projects than others. When looking for an ESP32 development board there are several aspects you need to take into account:

- USB-to-UART interface and voltage regulator circuit. Most full-featured development boards have these two features. This is important to easily connect the ESP32 to your computer to upload code and apply power.

- BOOT and RESET/EN buttons to put the board in flashing mode or reset (restart) the board. Some boards don’t have the BOOT button. Usually, these boards go into flashing mode automatically.

- Pin configuration and the number of pins. To properly use the ESP32 in your projects, you need to have access to the board pinout (like a map that shows which pin corresponds to which GPIO and its features). So make sure you have access to the pinout of the board you’re getting. Otherwise, you may end up using the ESP32 incorrectly.

- Antenna connector. Most boards come with an onboard antenna for Wi-Fi signal. Some boards come with an antenna connector to optionally connect an external antenna. Adding an external antenna increases your Wi-Fi range.

- Battery connector. If you want to power your ESP32 using batteries, there are development boards that come with connectors for LiPo batteries—this can be handier. You can also power a “regular” ESP32 with batteries through the power pins.

- Extra hardware features. There are ESP32 development boards with extra hardware features. For example, some may come with a built-in OLED display, a LoRa module, a SIM800 module (for GSM and GPRS), a battery holder, a camera, or others.

#### What is the best ESP32 development board for beginners?

For beginners, we recommend an ESP32 board with a vast selection of available GPIOs, and without any extra hardware features. It’s also important that it comes with voltage regular and USB input for power and upload code.

In most of our ESP32 projects, we use the ESP32 DEVKIT DOIT board, and that’s the one we recommend for beginners. There are different versions of this board with a different number of available pins (30, 36, and 38)—all boards work in a similar way.

Where to Buy?

You can check the following link to find the ESP32 DEVKIT DOIT board in different stores:

- ESP32 DEVKIT DOIT board

Other similar boards with the features mentioned previously may also be a good option like the Adafruit ESP32 Feather, Sparkfun ESP32 Thing, NodeMCU-32S, Wemos LoLin32, etc.

### ESP32 DEVKIT DOIT

In this article, we’ll be using the ESP32 DEVKIT DOIT board as a reference. If you have a different board, don’t worry. The information on this page is also compatible with other ESP32 development boards.

The picture below shows the ESP32 DEVKIT DOIT V1 board, version with 36 GPIO pins.

### Specifications – ESP32 DEVKIT V1 DOIT

The following table shows a summary of the ESP32 DEVKIT V1 DOIT board features and specifications:

| Number of cores | 2 (dual core) |
| --- | --- |
| Wi-Fi | 2.4 GHz up to 150 Mbits/s |
| Bluetooth | BLE (Bluetooth Low Energy) and legacy Bluetooth |
| Architecture | 32 bits |
| Clock frequency | Up to 240 MHz |
| RAM | 512 KB |
| Pins | 30, 36, or 38 (depending on the model) |
| Peripherals | Capacitive touch, ADC (analog to digital converter), DAC (digital to analog converter), I2C (Inter-Integrated Circuit), UART (universal asynchronous receiver/transmitter), CAN 2.0 (Controller Area Netwokr), SPI (Serial Peripheral Interface), I2S (Integrated Inter-IC Sound), RMII (Reduced Media-Independent Interface), PWM (pulse width modulation), and more. |
| Built-in buttons | RESET and BOOT buttons |
| Built-in LEDs | built-in blue LED connected to GPIO2; built-in red LED that shows the board is being powered |
| USB to UART bridge | CP2102 |

This particular ESP32 board comes with 36 pins, 18 on each side. The number of available GPIOs depends on your board model.

To learn more about the ESP32 GPIOs, read our GPIO reference guide: ESP32 Pinout Reference: Which GPIO pins should you use?

It comes with a microUSB interface that you can use to connect the board to your computer to upload code or apply power.

It uses the CP2102 chip (USB to UART) to communicate with your computer via a COM port using a serial interface. Another popular chip is the CH340. Check what’s the USB to UART chip converter on your board because you’ll need to install the required drivers so that your computer can communicate with the board (more information about this later in this guide).

This board also comes with a RESET button (may be labeled EN) to restart the board and a BOOT button to put the board in flashing mode (available to receive code). Note that some boards may not have a BOOT button.

It also comes with a built-in blue LED that is internally connected to GPIO 2. This LED is useful for debugging to give some sort of visual physical output. There’s also a red LED that lights up when you provide power to the board.

## ESP32 GPIOs Pinout Guide

The ESP32 chip comes with 48 pins with multiple functions. Not all pins are exposed in all ESP32 development boards, and some pins should not be used. The ESP32 DEVKIT V1 DOIT board usually comes with 36 exposed GPIOs that you can use to connect peripherals.

Power Pins

Usually, all boards come with power pins: 3V3, GND, and VIN. You can use these pins to power the board (if you’re not providing power through the USB port), or to get power for other peripherals (if you’re powering the board using the USB port).

General Purpose Input Output Pins (GPIOS)

Almost all GPIOs have a number assigned and that’s how you should refer to them—by their number.

With the ESP32 you can decide which pins are UART, I2C, or SPI – you just need to set that on the code. This is possible due to the ESP32 chip’s multiplexing feature that allows to assign multiple functions to the same pin.

If you don’t set them on the code, the pins will be configured by default as shown in the figure below (the pin location can change depending on the manufacturer). Additionally, there are pins with specific features that make them suitable or not for a particular project.

We have a detailed guide dedicated to the ESP32 GPIOs that we recommend you read: ESP32 Pinout Reference Guide. It shows how to use the ESP32 GPIOs and explains what are the best GPIOs to use depending on your project.

The placement of the GPIOs might be different depending on your board model. However, usually, each specific GPIO works in the same way regardless of the development board you’re using (with some exceptions). For example, regardless of the board, usually GPIO5 is always the VSPI CS0 pin, GPIO 23 always corresponds to VSPI MOSI for SPI communication, etc.